Proposal to Install a Southern Railway Semaphore Signal at the Signal Box

##### Introduction

The disused Billingshurst signal box, which is celebrating its 150th anniversary this year, was installed at the museum about 10 years ago.  Rather than the box being a static exhibit the emphasis has been on having a volunteer demonstrate the process of running trains, which was by passing them along a route from signal box to signal box, and setting signals for the drivers accordingly.

For demonstration purposes we obviously lack trains, signals and neighbouring signal boxes.  However a recent improvement has been the addition of a microprocessor simulator to mimic the movement of trains by controlling the lamps on the track circuit diagram that, along with the lever frame, forms the centre-piece of the signal box, and to simulate the bell code communications with an imaginary signaller in the next signal box along the line.

The signal box levers can all be operated, and frequently are especially by enthusiastic younger visitors, but other than a scale model signal in the box there is nothing to show the results of their efforts.  Fortunately when the signal box was delivered to the museum, we were also provided with a signal removed from nearby, this dates from the Southern Railway era (1923-1947) and presently lies in the white pit area.  We now propose installing this signal adjacent to the signal box and operating it from one of the signal levers, either mechanically or electrically.  The signal is of the old semaphore type with a moving arm, this iconic variety has been in use on the railways in various forms since the 1840s but is now almost completely obsolete and worthy of preservation.

Signal as delivered to the white pit area of the museum

Southern Railway semaphore signal at Amberley station

##### Required Works

The main tasks are as follows:

- Refurbishment of the signal arm, this includes:

Preparation of surfaces and applying coatings

Fitting of any parts that would be difficult once the signal is erected

- Preparation of the signal site:

Excavation, either by hand or using an auger, to a depth of 0.9-1.2m

Formation of a concrete base with a 30Mpa mix

- Moving the signal to site and installing it:

Using a flat bed lorry with a HIAB crane

- Connecting the signal to the signal box:

Either a mechanical link from a signal box lever, or an electrical actuation system

##### Costs

The refurbishment would be carried out using volunteer labour, although some low cost preparation materials would be required.  The signal as delivered is missing the signal arm, a lamp, a counterweight and some minor fittings, although we have already obtained a donated replacement signal arm.  Structurally the signal looks sound.  More significant would be the cost of the paint eg a 2-pack epoxy (white and black).

Likewise the operating mechanism can be installed with volunteer labour although some low to moderate cost items would be required.  A mechanical method (as-per the prototype), only requiring a length of galvanized wire and some small pulleys, would probably be cheaper than an electrical actuator, although would involve more volunteer labour.

It should be noted that the cost of installing the signal in a working condition rather than as a static exhibit requires only a relatively small increase in overall cost and is therefore recommended.

Contractors would be required for the following:

Excavation using an auger

Hire of a HIAB lorry with driver/operator